Great question! Biaxially Oriented Polypropylene (BOPP) film is one of the most important PP applications worldwide, used in packaging, labeling, and industrial films. Producers like Braskem offer several homopolymer PP grades for BOPP because the process and end-use requirements demand different balances of processability, mechanical properties, and optical performance. Here’s why multiple grades exist:

## 🔑 Why Multiple Homopolymer PP Grades Exist in BOPP

### 1. Processing Requirements

- Melt Flow Index (MFI):

  - Low MFI grades → higher viscosity, better melt strength, ideal for stable film extrusion and orientation.

  - High MFI grades → easier flow, faster throughput, suited for thinner films and high-speed lines.

- BOPP requires uniform melt rheology to ensure consistent stretching in both machine and transverse directions. Different grades are tuned for line speed, film thickness, and orientation ratios.

### 2. Mechanical Property Needs

- Homopolymer PP provides high stiffness, tensile strength, and dimensional stability, which are critical for BOPP films.

- Multiple grades allow fine-tuning of:

  - Modulus and tensile strength (important for packaging films that must resist tearing).

  - Shrinkage and orientation behavior (to ensure balanced properties in both directions).

  - Barrier properties (for moisture resistance in food packaging).

### 3. Application Diversity

- Food packaging films: Require clarity, stiffness, and barrier properties → mid-range homopolymer grades.

- Label films: Need dimensional stability and printability → specialized grades with optimized surface properties.

- Industrial films (lamination, adhesive tapes): Require toughness and high productivity → high MFI grades.

- Metallizable films: Need excellent barrier and adhesion to aluminum → grades with tailored crystallinity and surface energy.

### 4. Additive & Performance Packages

- Different grades incorporate antioxidants, slip agents, antistatic agents, or nucleating agents depending on whether the film is for food contact, labeling, or industrial use.

- Some grades are optimized for pigment dispersion or metallization compatibility, which is critical for decorative and barrier films.

## ⚖️ Summary

- Homopolymer PP grades are chosen for strength, stiffness, and barrier performance in BOPP films.

- Multiple grades exist because BOPP applications range from clear food packaging films to metallized industrial laminates, each requiring a different balance of melt flow, orientation behavior, mechanical strength, and surface properties.

- By offering several grades, producers ensure that customers can match the PP resin to their specific BOPP process and end-use performance needs.

👉 I can also prepare a comparison chart of Braskem’s BOPP PP grades (melt flow, orientation properties, barrier performance, typical applications) so you can see how each one is positioned in the portfolio. Would you like me to build that chart?