Great question! Cast film extrusion is a widely used process for polypropylene (PP), especially in packaging, labeling, and industrial films. Producers like Braskem offer several homopolymer and random copolymer PP grades because cast film applications demand different balances of processability, mechanical strength, optical properties, and end-use performance. Here’s why multiple grades exist:

## 🔑 Why Multiple PP Grades Exist in Cast Film Extrusion

### 1. Processing Requirements

- Melt Flow Index (MFI):

  - Low MFI grades → higher viscosity, better melt strength, ideal for thicker films and stable extrusion.

  - High MFI grades → easier flow, faster throughput, suited for thin films and high-speed lines.

- Cast film extrusion requires uniform melt flow and cooling behavior to ensure consistent film thickness and surface quality. Different grades are tuned for line speed, film gauge, and cooling conditions.

### 2. Mechanical Property Needs

- Homopolymer PP (HOMO):

  - Provides high stiffness, tensile strength, and heat resistance.

  - Multiple grades allow fine-tuning between productivity and mechanical strength.

  - Common in films that need rigidity and dimensional stability (e.g., lamination base films, industrial packaging).

- Random Copolymer PP (RACO):

  - Offers clarity, gloss, and flexibility.

  - Different grades balance transparency with mechanical performance.

  - Used in consumer-facing packaging where aesthetics and flexibility matter (e.g., food wraps, medical films).

### 3. Application Diversity

- Food packaging films: Require clarity and heat resistance → random copolymer grades.

- Industrial packaging (sacks, heavy-duty wraps): Need stiffness and strength → homopolymer grades.

- Lamination films: Require dimensional stability and printability → homopolymer grades.

- Consumer packaging (clear bags, wraps): Require gloss and flexibility → random copolymer grades.

- Medical packaging: Needs transparency and chemical resistance → random copolymer grades.

### 4. Additive & Performance Packages

- Different grades incorporate antioxidants, slip agents, antistatic agents, or UV stabilizers depending on whether the film is for food contact, outdoor use, or industrial packaging.

- Some grades are optimized for pigment dispersion or recycling compatibility, which is increasingly important in cast film production.

## ⚖️ Summary

- Homopolymer PP grades → chosen for rigidity, strength, and dimensional stability.

- Random copolymer PP grades → chosen for clarity, gloss, and flexibility.

- Multiple grades exist because cast film applications range from clear food wraps to heavy-duty industrial laminates, each requiring a different balance of flowability, stiffness, toughness, and optical properties.

👉 I can also prepare a comparison chart of Braskem’s cast film PP grades (melt flow, clarity, tensile strength, typical applications) so you can see how each one is positioned in the portfolio. Would you like me to build that chart?