Great question! Raffia extrusion is one of the most traditional and large-scale applications of polypropylene (PP), especially in woven sacks, ropes, and agricultural fabrics. Producers like Braskem offer several homopolymer PP grades for raffia because the process and end-use products demand different balances of processability, mechanical strength, and cost efficiency. Here’s why:

## 🔑 Why Multiple Homopolymer PP Grades Exist in Raffia Extrusion

### 1. Processing Requirements

- Melt Flow Index (MFI):

  - Low MFI grades → higher viscosity, better melt strength, ideal for stable film extrusion and stretching into tapes.

  - High MFI grades → easier flow, faster throughput, suited for high-speed lines and finer tapes.

- Raffia extrusion involves film extrusion + slit + stretching into tapes, so different grades ensure stability across line speeds and tape dimensions.

### 2. Mechanical Property Needs

- Homopolymer PP provides high stiffness and tensile strength, which are critical for woven sacks and ropes.

- Multiple grades allow fine-tuning of:

  - Tensile modulus (for strong tapes).

  - Elongation and shrinkage behavior (important during stretching).

  - Durability under load (for heavy-duty sacks and ropes).

### 3. Application Diversity

- Woven sacks for cement, grains, sugar: Need high strength and dimensional stability → low MFI homopolymer grades.

- Agricultural fabrics (shade nets, ground covers): Require balance of stiffness and flexibility → mid-range grades.

- Ropes and twines: Need toughness and abrasion resistance → specialized homopolymer grades with optimized crystallinity.

- Lightweight packaging tapes: Require high productivity and thinner tapes → high MFI grades.

### 4. Additive & Performance Packages

- Different grades incorporate UV stabilizers (for outdoor agricultural use), antioxidants (for long-term durability), or pigment dispersion optimization (for colored raffia products).

- Some grades are tailored for recycling compatibility, important in woven sack production.

## ⚖️ Summary

- Homopolymer PP grades are chosen for strength, stiffness, and processability in raffia extrusion.

- Multiple grades exist because raffia applications range from heavy-duty woven sacks to lightweight packaging tapes, each requiring a different balance of melt flow, tensile properties, and stabilization.

- By offering several grades, producers ensure that customers can match the PP resin to their specific raffia process and end-use performance needs.

👉 I can also prepare a comparison chart of Braskem’s raffia extrusion PP grades (melt flow, tensile strength, UV stability, typical applications) so you can see how each one is positioned in the portfolio. Would you like me to build that chart?