## 🔑 Why Multiple Grades Exist in Thermoforming

### 1. Processing Behavior

- Melt Flow Index (MFI):

  - Low MFI grades → higher viscosity, better sheet stability during heating and forming, ideal for large trays or industrial packaging.

  - High MFI grades → easier flow, faster cycle times, suited for thin-walled or complex thermoformed parts.

- Thermoforming requires uniform sheet extrusion and controlled sag resistance, so different grades ensure stability across part sizes.

### 2. Mechanical Property Requirements

- Homopolymer PP (HOMO):

  - Provides high stiffness, rigidity, and heat resistance.

  - Multiple grades allow tuning between productivity and mechanical strength.

  - Common in rigid food packaging, industrial trays, and technical thermoformed parts.

- Random Copolymer PP (RACO):

  - Offers clarity, gloss, and flexibility.

  - Different grades balance transparency with mechanical performance.

  - Used in clear cups, lids, and medical packaging where aesthetics and visibility are critical.

- Heterophasic Copolymer PP (HECO):

  - Provides impact resistance and toughness, especially at low temperatures.

  - Multiple grades balance toughness with stiffness depending on whether the thermoformed part must resist drops, stacking, or cold-chain logistics.

  - Common in heavy-duty packaging, automotive liners, and industrial containers.

### 3. Application Diversity

- Food packaging (cups, trays, lids):

  - Needs clarity and heat resistance → random copolymer or homopolymer grades.

- Industrial packaging (crates, pallets, containers):

  - Requires toughness and drop resistance → heterophasic copolymer grades.

- Automotive thermoformed parts (liners, panels):

  - Need impact resistance and durability → heterophasic copolymer grades.

- Medical packaging (blister packs, trays):

  - Requires transparency and chemical resistance → random copolymer grades.

### 4. Additive & Performance Packages

- Grades differ in stabilization packages (UV resistance, antistatic, slip agents) depending on whether the thermoformed part is for outdoor use, food contact, or medical applications.

- Some grades are optimized for pigment dispersion or recycling compatibility, which is increasingly important in thermoformed packaging.

## ⚖️ Summary

- Homopolymer PP grades → chosen for rigidity, heat resistance, and dimensional stability.

- Random copolymer PP grades → chosen for clarity, gloss, and consumer-facing packaging.

- Heterophasic copolymer PP grades → chosen for impact strength, toughness, and durability.

- Multiple grades exist because thermoforming applications range from clear food cups to industrial crates, each requiring a different balance of flowability, stiffness, toughness, and optical properties.

👉 I can also prepare a comparison chart of Braskem’s thermoforming PP grades (melt flow, impact strength, clarity, and typical applications) so you can see how each one is positioned in the portfolio. Would you like me to build that chart?